| Altimeter and statics system test in accordance with LTH 40 in agreement with FAR part 43 App.E and F | Form 40.3 | REGISTRATION: Characteristic: |
| --- | --- | --- |
|  | REVISION Revision 0 | AIRCRAFT TYPE: SERIAL No.: |
|  |  | Project No.: |
|  | Page 1 of 1 | DATE: Date: |
| 1. General information. Place of execution; Field altitude ft. Airfield information; ALt. No.1; stby; Alt. No.2. QFE:. QNH:. Tolerance +/- 20 ft. Altimeter S/N. 2. Statics system leak test. A) without pressurized cabin: Diff. Pressure at the altimeter 1000 ft over QFE. b) with pressurized cabin: Altimeter at the max. Kabinendiff.- pressure over QFE Used max. Cabin Alt:. MAX. pressure drop after 1min 100 ft Nr.1 Ind: Nr.2 Ind:. 3. Scale error. Checkpoints; lndication in ft; Tolerance; Encoder. in ft; No.1; stby; No.2; +/- ft; +/- 125 ft. - 1000; 20. 0; 20. 500; 20. 1000; 20. 1500; 25. 2000; 30. 3000; 30. 4000; 35. 4100; Encoder with Gilham Code only. 8000; 60. 10000; 80. 12000; 90. 14000; 100. 15700; Encoder with Gilham Code only. 16000; 110. 18000; 120. 20000; 130. 22000; 140. 25000; 155. 30000; 180. 31000; Encoder with Gilham Code only. 35000; 205. 40000; 230. 45000; 255. 50000; 280. 4. Hysteresis test. Alt 1; stby; Alt2; Tol ft. Housing leak test: At 18000 ft max. 100 ft/min; - 100. Testpoint 1 50% max. Alt; +/- 75. Testpoint 2 40% max. Alt; +/- 75. Elast. effect. with daily pressure; +/- 30.; 5. Barometric scale errors. Pressure; Pressure; Altitude. in ft; Indicator Toler. +/- 25 ft. inch of Hg; hPa; for inHg; for hPa; No.1; stby; No.2. 28,10; 952; - 1727; - 1715. 28,50; 965; - 1340; - 1334. 29,00; 982; - 863; - 864. 29,50; 999; - 392; - 391. 29,92; 1013; 0; 0. 30,50; 1033; 531; 531. 30,90; 1046; 893; 883. 30,99; 1050; 974; 989. 6. Friction (750/min ft). Checkpoints; lndication in ft; Tolerance. in ft; No.1; stby; No.2; +/- ft. 1000; 70. 2000; 70. 3000; 70. 5000; 70. 10000; 80. 15000; 90. 20000; 100. 25000; 120. 30000; 140. 35000; 160. 40000; 180. 50000; 250. 7. Conclusion: All lines connected/original state. Mechanic.:. Insp.:; Stamp. Air DATA test set information. Manufacturer:. Model:. Serial NUMBER:. DATE OF Calibration:. Accuracy Specification:.. |