| MODE S TRANSPONDER TESTREPORT | Formblatt 40.2 B | REGISTRATION: Kennzeichen: |
| --- | --- | --- |
|  | REVISION Revision 0 | AIRCRAFT TYPE: SERIAL-No.: |
|  | Seite 1 von 4 | PROJECT-No.: |
| Performed by: |  | DATE: Datum: |
| afa . Units under test; #1; #2; Testset. Manufacturer; Manufacturer. Type; Type. Partnumber; Serial Number. Serial Number; Calibration. Mod Status. Elementary Surveillance capable. Enhanced Surveillance capable. Results. Tests; #1; #2; Tolerance; Remarks. Transmitterfrequency; Mhz; Mhz; 1090 ±1 Mhz. Sensitivity (MTL); lower Antenna; dBm; dBm; -73 ±4 dBm. upper Antenna; dBm; dBm; -73 ±4 dBm. Power; lower Antenna; dBm; dBm; 48,5< ERP <57,0. upper Antenna; dBm; dBm; 48,5< ERP <57,0. SLS XPDR Reply; SLS 0dB; %; %; max. 1%. SLS –9dB; %; %; min. 90%. Mode S address; _____________; _____________; Transponder must not reply to any other than its assigned address. ○ pass; ○ fail; ○ pass; ○ fail. Mode S diversity Isolation; dB; dB; >20dB; selected antenna >20dBm> nonselected. ATCRBS/Mode S all-call, all-call address; _____________; _____________; verify correct all-call address is reported. ATCRBS-only all-call interrogation; ○ pass; ○ fail; ○ pass; ○ fail; verify no reply. MTL difference (modeA-modeC); dBm; dBm; ±1 dBm. Squitter; ○ pass; ○ fail; ○ pass; ○ fail; approx. once/second. Auto Test (if available); ○ pass; ○ fail; ○ pass; ○ fail. * Ident; ○ pass; ○ fail; ○ pass; ○ fail; on for ca. 20 seconds. * Pilots Code; ○ pass; ○ fail; ○ pass; ○ fail. * Altitude Code; ○ pass; ○ fail; ○ pass; ○ fail; see main altimeter at 1013 mb. ** Mode S formats; ○ pass; ○ fail; ○ pass; ○ fail; refer to pages 2 to 4. * No FAR 43 Appendix F requirement. ** Fill in table on page 2/4 Mode S uplink formats 0, 4, 5, 11, 16, 17, 20, 21, 24 Check for which uplinkformats transponder is equipped Check for correct reply, refer to attached downlink format table #1; #2. DF; =0; DF; =17; DF; =0; DF; =17. VS; =__________; CA; =__________; VS; =__________; CA; =__________. CC; =__________; AA; =__________; CC; =__________; AA; =__________. SL; =__________; ME; =__________; SL; =__________; ME; =__________. RI; =__________; PI; =__________; RI; =__________; PI; =__________. AC; =__________; AC; =__________. Address; =__________; DF; =20; Address; =__________; DF; =20. FS; =__________; FS; =__________. DF; =4; DR; =__________; DF; =4; DR; =__________. FS; =__________; UM; =__________; FS; =__________; UM; =__________. DR; =__________; AC; =__________; DR; =__________; AC; =__________. UM; =__________; MB; =__________; UM; =__________; MB; =__________. AC; =__________; Address; =__________; AC; =__________; Address; =__________. Address; =__________; Address; =__________. DF; =21; DF; =21. DF; =5; FS; =__________; DF; =5; FS; =__________. FS; =__________; DR; =__________; FS; =__________; DR; =__________. DR; =__________; UM; =__________; DR; =__________; UM; =__________. UM; =__________; ID; =__________; UM; =__________; ID; =__________. ID; =__________; MB; =__________; ID; =__________; MB; =__________. Address; =__________; Address; =__________; Address; =__________; Address; =__________. DF; =11; DF; =24; DF; =11; DF; =24. CA; =__________; KE; =__________; CA; =__________; KE; =__________. AA; =__________; ND; =__________; AA; =__________; ND; =__________. PI; =__________; MD; =__________; PI; =__________; MD; =__________. Address; =__________; Address; =__________. DF; =16; DF; =16. VS; =__________; VS; =__________. SL; =__________; SL; =__________. RI; =__________; RI; =__________. AC; =__________; AC; =__________. MV; =__________; MV; =__________. Address; =__________; Address; =__________. All tests where performed in accordance with ACG-LTH 40, all limits according to FAR 43 Appendix F. The Transponder(s) are in a airworthy condition.; yes; no. Date and Location; Performed by. # Mode S formats and field contents Downlink Formats. FORMAT NO.; DF; CONTENTS; DESCRIPTION. 0; 0 0000; VS, CC, SL, RI, AC, AP; Short Air - Air Surveillance. 4; 0 0100; FS, DR, UM, AC, AP; Surveillance, Altitude Reply. 5; 0 0101; FS, DR, UM, ID, AP; Surveillance, Identity Reply. 11; 0 1011; CA, AA, PI; All Call Reply/Squitter. 16; 1 0000; VS, SL, RI, AC, MV, AP; Long Air - Air Surveillance. 17; 1 0001; CA, AA, ME, PI; Extended Squitter. 20; 1 0100; FS, DR, UM, AC, MB, AP; Comm-B, Altitude Reply. 21; 1 0101; FS, DR, UM, ID, MB, AP; Comm-B, Identity Reply. 24; 1 1xxx; KE, ND, MD, AP; Comm-D (ELM). Field contents Field; Bits; Contents. AA; 24; The aircraft address announced in the clear, used in DF: 11, 17. AC; 13; The altitude code, used in DF: 0, 4, 16, 20. All bits are Zeros if altitude information is not available.. Contains metric altitude if M-Bit (26) is One.. AP; 24; Parity overlaid on the address appears at the end of all transmissions up- and downlink except DF: 11. CA; 3; Squitter or All Call Reply, to report Transponder Capability. Codes 4-7 are for squitter only. Used DF: 11. 0; Level 1, no communication capability (survaillance only), XPDR transmits DF: 0, 4, 5, 11. 1; Level 2, Comm-A and Comm-B capability, XPDR transmits DF: 0, 4, 5, 11, 20, 21. 2; Level 3, Comm-A, Comm-B and uplink ELM capability, XPDR transmits DF: 0, 4, 5, 11, 20, 21, 24. 3; Level 4, Comm-A, Comm-B up- and downlink ELM capability, transmits DF: 0, 4, 5, 11, 20, 21, 24. 4; Level 2, 3 or 4; ability to set code 7 and is on ground. 5; Level 2, 3 or 4; ability to set code 7 and is on airborne. 6; Level 2, 3 or 4; ability to set code 7. 7; DR≠0 or FS=3, 4 or 5. CC; 1; Indicates XPDR has ability to support crosslink capability, used in DF: 0. DF; 5; The first field in all downlink formats is the transmission descriptor.. DR; 5; Requests extraction of downlink and appears in DF: 4, 5, 20, 21; codes 1-15 take priority over 16-31. 0; No downlink request.. 1; Request to send Comm-B message (B-Bit set).. 2; TCAS information available.. 3; TCAS information available and request to send Comm-B message.. 4; Comm-B broadcast #1 available.. 5; Comm-B broadcast #2 available.. 6; TCAS information and Comm-B broadcast #1 available.. 7; TCAS information and Comm-B broadcast #2 available.. 8-15; Not assigned.. 16-31; Request to send n-segments signified by 15+n.. FS; 3; Reports flight status of A/C and is used in DF: 4, 5, 20, 21. Code; Alert; SPI; airborne; on GND. 0; no; no; yes; no. 1; no; no; no; yes. 2; yes; no; yes; no. 3; yes; no; no; yes. 4; yes; yes; either. 5; no; yes; either. 6,7; Not assigned. ID; 13; The 4096 identification code as set by the pilot in DF: 5, 21. Field; Bits; Contents. KE; 1; Defines the content of the ND and MD fields in Comm-D replies, DF:24.. MB; 56; Messages to be transmitted to the interrogator, part of the Comm-B replies, DF: 20, 21. Contains Data Link,. Capability reports or A/C Identification Reports or TCAS Resolution Advisory Reports.. MD; 80; Comm-D field, part of DF: 24, contains one segment of a sequence transmitted by XPDR in the ELM mode.. ME; 56; Extended squitter message, part of DF: 17, contains broadcast messages.. MV; 56; Part of the long air-air surveillance reply, DF: 16, contains information used in air-air exchanges. (Coordination Reply Messages). ND; 4; Provides the segment number transmitted in a downlink ELM and is part of a Comm-D reply, DF: 24. PI; 24; Contains the parity overlaid on the interrogator identity code, DF: 11, Mode S Only All Call.. RI; 4; Information in DF: 0, 16. 0; No on-board TCAS.. 1; Not assigned.. 2; On-board TCAS with resolution capability inhibited.. 3; On-board TCAS with vertical-only resolution capability.. 4; On-board TCAS with vertical and horizontal resolution capability.. 5-7; Not assigned.. 8; No maximum airspeed data available.. 9; Airspeed is ≤75kts.. 10; Airspeed is >75kts and ≤150kts.. 11; Airspeed is >150kts and ≤300kts.. 12; Airspeed is >300kts and ≤600kts.. 13; Airspeed is >600kts and ≤1200kts.. 14; Airspeed is >1200kts.. 15; Not assigned.. SL; 3; Reports the current operating sensitivity level of TCAS, is part of air-air surveillance replies, DF: 0, 16. Has no meaning for A/C with RI=0 ( no on board capability to generate resolution advisories).. 0; No TCAS sensitivity level reported. 1; TCAS operates at sensitivity level 1.. 2; TCAS operates at sensitivity level 2.. 3; TCAS operates at sensitivity level 3.. 4; TCAS operates at sensitivity level 4.. 5; TCAS operates at sensitivity level 5.. 6; TCAS operates at sensitivity level 6.. 7; TCAS operates at sensitivity level 7.. 6; Contains XPDR status readouts in DF: 4, 5, 20, 21.. VS; 1; Indicates A/C is airborne when Zero or on ground when One.. |